PARKINSON’S TREMOR DETECTION SYSTEM
by

HARI SAI SENTHIL KUMAR
Roll No: 24BLC1108

ANIRUDHAN
Roll No: 24BLC1335

A project report submitted to

Dr. GUGAPRIYA

SCHOOL OF ELECTRONICS ENGINEERING

in partial fulfilment of the requirements for the course of

BECE204L – MICROPROCESSORS AND MICROCONTROLLERS

B.Tech. ELECTRONICS AND COMMUNICATION ENGINEERING

Vandalur – Kelambakkam Road
Chennai – 600127
NOVEMBER 2025

# BONAFIDE CERTIFICATE

Certified that this project report entitled “PARKINSON’S TREMOR DETECTION SYSTEM” is a bonafide work of HARI SAI SENTHIL KUMAR (24BLC1108) and ANIRUTHAN (24BLC1335), who carried out the project work under my supervision and guidance for BECE204L – MICROPROCESSORS AND MICROCONTROLLERS.

Dr. GUGAPRIYA
Associate Professor
School of Electronics Engineering (SENSE)
VIT Chennai

# ABSTRACT

Parkinson’s disease is a neurological disorder characterized by involuntary tremors. This project focuses on detecting tremors within the frequency range of 4–6 Hz using an MPU6050 sensor and ESP-32. The gyroscope data from MPU6050 is processed using the Fast Fourier Transform (FFT) algorithm to identify dominant frequencies. If the measured frequency lies between 4–6 Hz and exceeds a certain threshold, the system flags it as a tremor. This low-cost, real-time detection system can assist in monitoring tremor activity and potentially aid in Parkinson’s diagnosis and treatment monitoring.

# MATERIALS USED

• ESP32 Dev Module

• MPU6050 Sensor Module

• Connecting Wires

• Breadboard

• USB Cable

• Buzzer

• LCD Display

# BLOCK DIAGRAM

CIRCUIT DIAGRAM

# WORKING PRINCIPLE

- The MPU6050 sensor captures hand motion data in the form of angular velocity along the X, Y, and Z axes.

- The ESP32 microcontroller continuously samples 128 gyroscopic readings at a rate of 100 Hz.

- The collected time-domain data is processed using Fast Fourier Transform (FFT) to convert it into the frequency domain.
The dominant frequency component of the motion is identified from the FFT output.

- If the dominant frequency lies within the 4–6 Hz range and the magnitude exceeds a predefined threshold, the system classifies the motion as a tremor event.

- The LCD display shows real-time status messages such as “Normal”, “Pre-Tremor Detected”, or “Tremor Detected” based on the analysis.

- The Serial Monitor simultaneously logs the detected frequency and tremor status, while the buzzer provides an audible alert during tremor detection.

# ARDUINO CODE

#include <Wire.h>

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <arduinoFFT.h>

#include <LiquidCrystal_I2C.h>

#define BUZZER_PIN 25

#define MPU_SDA 21

#define MPU_SCL 22

#define LCD_SDA 18

#define LCD_SCL 19

Adafruit_MPU6050 mpu;

TwoWire I2CMPU = TwoWire(0);

TwoWire I2CLCD = TwoWire(1);

LiquidCrystal_I2C lcd(0x27, 16, 2);

#define SAMPLES 128

#define SAMPLING_FREQUENCY 100.0

ArduinoFFT<double> FFT = ArduinoFFT<double>();

double vReal[SAMPLES];

double vImag[SAMPLES];

#define TREMOR_MIN_HZ 4.0

#define TREMOR_MAX_HZ 6.0

#define TREMOR_THRESHOLD 10.0

unsigned long lastSampleMicros = 0;

int sampleCount = 0;

const unsigned long samplingInterval = 1000000 / SAMPLING_FREQUENCY;

void setup() {

Serial.begin(115200);

pinMode(BUZZER_PIN, OUTPUT);

ledcSetup(0, 2000, 8);

ledcAttachPin(BUZZER_PIN, 0);

ledcWrite(0, 0);

I2CMPU.begin(MPU_SDA, MPU_SCL, 100000);

I2CLCD.begin(LCD_SDA, LCD_SCL, 100000);

lcd.begin(16, 2);

lcd.backlight();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Parkinson's Test");

lcd.setCursor(0, 1);

lcd.print("Initializing...");

delay(1500);

lcd.clear();

if (!mpu.begin(0x68, &I2CMPU)) {

Serial.println("MPU6050 not found. Check wiring!");

lcd.print("MPU Error!");

while (1);

}

Serial.println("MPU6050 Connected!");

lcd.print("MPU Ready");

delay(1000);

lcd.clear();

}

void loop() {

if (sampleCount < SAMPLES) {

if (micros() - lastSampleMicros >= samplingInterval) {

lastSampleMicros = micros();

sensors_event_t a, g, temp;

mpu.getEvent(&a, &g, &temp);

vReal[sampleCount] = g.gyro.x;

vImag[sampleCount] = 0;

sampleCount++;

}

return;

}

FFT.windowing(vReal, SAMPLES, FFT_WIN_TYP_HAMMING, FFT_FORWARD);

FFT.compute(vReal, vImag, SAMPLES, FFT_FORWARD);

FFT.complexToMagnitude(vReal, vImag, SAMPLES);

double peakFrequency = 0;

double peakMagnitude = 0;

for (int i = 1; i < (SAMPLES / 2); i++) {

double freq = (i * SAMPLING_FREQUENCY) / SAMPLES;

if (vReal[i] > peakMagnitude) {

peakMagnitude = vReal[i];

peakFrequency = freq;

}

}

Serial.print("Dominant Freq: ");

Serial.print(peakFrequency, 2);

Serial.print(" Hz  |  Magnitude: ");

Serial.println(peakMagnitude, 2);

if (peakFrequency >= TREMOR_MIN_HZ && peakFrequency <= TREMOR_MAX_HZ && peakMagnitude > TREMOR_THRESHOLD) {

Serial.println("Tremor Detected!");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("TREMOR DETECTED!");

ledcWriteTone(0, 2000);

} else if (peakFrequency >= TREMOR_MIN_HZ - 1 && peakFrequency <= TREMOR_MAX_HZ + 1 && peakMagnitude > (TREMOR_THRESHOLD / 2)) {

Serial.println("Pre-Tremor Detected!");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Pre-Tremor");

lcd.setCursor(0, 1);

lcd.print("Detected");

ledcWriteTone(0, 1000);

} else {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Normal");

ledcWriteTone(0, 0);

}

sampleCount = 0;

delay(1000);

}

# CONCLUSION

The Parkinson’s Tremor Detection System successfully identifies tremors in the 4–6 Hz frequency range. Using FFT-based signal analysis, the system provides a reliable, real-time indication of tremor activity. This prototype is low-cost and suitable for further enhancement using wireless communication or machine learning.

# REFERENCES

https://create.arduino.cc/projecthub/ - For programming the ESP-32 with Arduino IDE

https://www.instructables.com/ - For references to connect different components

https://components101.com/sensors/mpu6050-gyro-accelerometer-module - MPU6050 pinout and data sheet

https://www.arduino.cc/en/Reference/ArduinoFFT - For references to use Fast Fourier Transform to sample data from MPU6050 sensor

https://fritzing.org/ -  To make the circuit diagram with different components